LTE industrial router

- LTE industrial router

Instructions

Catalog

LTE industrial router	- 1 -

1. Interface description	- 4 -

2. Setup preparation	- 5 -

3. Internet settings	- 6 -

1. Switching of working mode	- 6 -

2. Internet access settings	- 7 -

3、 Wire break detection	- 13 -

4、 Dynamic domain name	- 13 -

5、 AT instruction	- 14 -

4. LAN settings	- 14 -

5. Wireless settings	- 15 -

1、 Basic information	- 15 -

2、 Wireless security	- 15 -

3、 Advanced settings	- 16 -

4、 List of wireless clients	- 16 -

5、 Wireless client MAC filtering	- 16 -

6. VPN settings	- 17 -

7. Network security	- 18 -

1. Firewall	- 18 -

2. Site control	- 19 -

3. Access control	- 19 -

4. Port blocking	- 20 -

8. System Services	- 21 -

1. Virtual service (port mapping)	- 21 -

2、DMZ	- 22 -

3. Serial port service	- 22 -

4. Remote maintenance platform	- 24 -

5. DO control	- 25 -

9. Equipment management	- 26 -

1. Equipment management	- 26 -

2. Time zone management	- 26 -

3. Backup/Recovery	- 27 -

4. Software upgrade	- 27 -

5. Restore the factory value	- 28 -

6. Password management	- 29 -

10. FAQ	- 29 -

- Interface description

Hardware interface description

| Project | Parameter |
| --- | --- |
| WAN port (WAN/LAN 1) | One 10/100M adaptive LAN/WAN port, with built-in isolation, supporting auto rollover (Auto MDI/MDIX); WAN port in standard routing mode and LAN port in other modes |
| LAN port (LAN2-4) | 3 10/100M self-adaptive LAN ports, supporting automatic turnover (Auto MDI/MDIX); built-in 1.5KV electromagnetic isolation protection |
| Serial port | 1 serial port, supporting data transmission and AT command |
| DO | 2 groups of independent single-wire DO control interfaces |
| Indicator light | 7 indicators, Wan, lan, sys, 4G, GPS |
| Antenna interface | 4 antenna interfaces, standard SMA antenna connection |
| USIM card interface | 2 SIM card holders |
| Power connector | Standard DC interface 5V-12V |

Indicator light status description

| Name | Status | Description |
| --- | --- | --- |
| WAN/LAN1-4 | Flicker | RJ45 port already connected |
|  | Put out | RJ45 port is not connected |
| 4G | Move | Dialing process in progress |
|  | Chang Liang | Dialing succeeded |
|  | Put out | LTE module or SIM card not recognized or hang-up status |
| System light (SYS) | Chang Liang | Power supply is normal |
|  | Slow flash | The system is operating normally |
| GPS | Chang Liang | The function is valid |
|  | Put out | This function is not available |

- Setup preparation

Connect the device

You can connect your computer to the router by following these steps.

Set the IP address of the computer

Before accessing the Web settings page, it is recommended that you set your computer to Obtain an IP Address Automatically and Obtain a DNS Server Address Automatically so that the router assigns the IP address automatically. If you need to assign a static IP address to the computer, you need to set the IP address of the computer and the IP address of the LAN port of the router in the same subnet (the default IP address of LAN port of the router is 192.168.10.1, and the subnet mask is 255.255.255.0).

Connect via WiFi

Detect the wireless network connection of the wireless router, and then click the'Connect 'button to establish the connection. The wireless password is: 12345678.

Verify connectivity between the computer and the router

When your computer shows that it has successfully obtained an IP, use the Ping command to confirm that connectivity between the computer and the router is successful.

For example, in the Windows 10 environment, execute the Ping command: Ping 192.168.10.1

If the screen displays the following, the computer has successfully established a connection with the router.

Log in to the router

Next, you will log on to the router Web settings page.

Enter "http://192.168.10.1" in the Web browser address bar, and enter the login user name and password in the pop-up box.

When logging in for the first time, please enter the default user name: admin and password: admin.

Go to the router Web settings pack

After logging in successfully, enter the Web settings page, and then you can set up and manage the router

- Internet settings

- 1. Switching of working mode

BC728C provides 4 working modes: 3G/4G wireless routing mode (default), standard wireless routing mode, wireless AP + wireless client bridge mode, and wireless AP + wireless client mode. After selecting the mode and submitting the confirmation, you can switch.

Except that the standard wireless routing mode has a physical Wan port, all physical network ports in other modes are LAN ports.

- 2. Internet access settings

- 2.1 3G/4G Wireless Routing Mode

The default mode is 3G/4G wireless routing mode. Insert the 3G/4G tariff card into the SIM card slot. After the device is powered on, dialing will automatically identify the operator network. If you are using a dedicated network card (not a generic APN card), you can also customize the network operator you need to select in the settings page.

The BC728C supports the insertion of dual SIM cards, but only supports the online operation of a single SIM card. It supports the automatic switching of the network using the backup SIM card when the primary SIM card network fails (the backup SIM card network needs to be enabled in the settings). When using only one SIM card, insert the SIM1 card slot.

Enter the 3G/4G setting page to configure dialing parameters

SIM card network switching settings

- Select SIM automatically: Enable is checked by default. When SIM1 dialing fails, it will switch to SIM2 dialing

- The signal strength is less than: the range is 1-31, and 31 is the strongest. When the signal is less than the set value for 10 consecutive queries, switch the SIM card network. Check SIM2 backup and set the use duration to automatically switch back to SIM1 network after expiration

- Manual assignment is mutually exclusive with automatic SIM selection. When only one SIM card is used, use SIM1

- Dialing failure: restart the device after the number of consecutive dialing failures reaches the set value

APN card parameter setting, as shown in the figure:

Cancel the option of automatically selecting the operator, and fill in the APN address, user name and password provided by the SIM card provider

If the APN card has no Internet access function, it needs to be maintained."WAN disconnection detection"A state that has been turned off, otherwise a network disconnect detection failure will cause intermittent network disconnections.

When using a universal card (mobile phone card, ordinary Internet of Things card), there is no need to set parameters, and the system automatically identifies the operator's network, as shown in the figure:

- 2.2 Standard Wireless Routing Mode

The standard wireless routing mode is wired routing, and the Wan port is plugged into the network cable to connect to the Internet. Support static IP, dynamic IP and PPPoE.

- 2.2.1 Static IP mode

Enter the WAN setting page, select the static address for the Internet access mode, fill in the four necessary parameters of IP (the IP address of the upper routing), subnet mask, default gateway and primary DNS, and click OK to submit. As shown in figure:

- 2.2.2 Dynamic IP acquisition method

Select the dynamic address for the Internet access mode, click OK to submit, and the router will automatically obtain the IP allocated by the upper routing.

- 2.2.3 PPPoE dialing method

Select PPPoE as the Internet access mode, enter the user name and password provided by the ISP, and click OK to submit.

- 2.3 Wireless AP + Client Bridge Mode

Wireless to wired, that is, the wireless network is transferred to the local wired LAN network by bridging the AP signal of the previous wireless router through the wireless wifi, and the device network connected to the LAN port is allocated IP by the upper wireless router and is in the same network segment.

In this mode, the device provides only the primary bridge function, and no other services are available.

First, set the local IP of the computer to 192.168.10.100, connect the LAN interface through the network cable, enter the router management interface, and select the wireless AP + client bridge mode.

1) Enter the LAN setting page, search for the wireless network to be bridged, check Enable, and click Search

2) Select the wireless network AP to be bridged in the pop-up page, and check the wireless AP.

3) After selecting the AP, the security mode and encryption type will be automatically matched. Fill in the password of the wireless AP and click OK to submit.

The wireless connection status can be viewed on the status page. After successful bridging, change the local IP acquisition mode of the computer back to the automatic acquisition mode to acquire the IP allocated by the bridged wireless router.

- 2.4 Wireless AP + Client Mode

Wireless AP + client mode can also be called wireless routing mode, that is, the local area network connects the AP signal of the previous wireless router to the Internet through the wifi client mode, and the local area network provides wired and wifi modes for terminal access.

The wifi client supports static IP, dynamic IP and PPPoE to connect to the previous wireless router to access the Internet. For static IP, dynamic IP, and PPPoE parameter settings, refer to the standard wireless routing parameter connection settings in Section 2.2.

For wireless connection to the AP, refer to the wireless AP search and connection steps in Section 2.3.

- Wire break detection

The disconnection detection is to periodically initiate a network detection action after the network is successfully connected, and if the continuous detection fails within the set check parameter value, the current network is judged to be abnormal or disconnected, and the automatic reconnection is executed to ensure the stability of the network.

If the accessed network cannot access the set host address, you need to change it to the accessible host IP/domain name, or turn off the disconnection detection, otherwise it will cause intermittent network disconnection (automatic reconnection).

All operating modes are enabled by default, except for the wireless client bridge mode.

- Host address: support IPv4, domain name, for multiple addresses; Semicolon connection

- Interval: the time interval for initiating network probe

- Retries: Number of consecutive probes

- Dynamic domain name

Integrated DDNS resolution, supporting multiple domain name providers such as oray, dyndns, 3322.org

The local interface can be resolved only when the IP obtained by the WAN external network interface is a legal public network IP.

- AT instruction

Used only in 3G/4G wireless routing mode to manually send module AT commands to query relevant information

- LAN settings

When there are multiple routers connected, if the LAN port IP addresses are the same, the IP address conflict will occur and the network cannot be connected. It is necessary to modify the LAN port IP address to solve the problem. Modify the IP address in the LAN setting page, such as 192.168.20.1 and 192.168.100.1.

DHCP service is used to dynamically allocate IP addresses for accessed terminals, modify the range of the last group of values, and limit the range of allocated IP addresses. After the DHCP server is disabled, the device will no longer automatically assign an IP address to the terminal connected to the LAN, and the terminal network card is required to assign an IP address in the same network segment as the device to connect, such as 192.168.10.2/24.

- Wireless settings

- Basic information

Wireless wifi name, wireless channel and hidden wifi can be set.

- SSID: wireless wifi name

- Do not broadcast SSID: Hide wireless wifi name

- Wireless channel: a wireless channel, which can be used in an environment with more channel interference to select a channel with less interference.

- Other options are recommended by default

- Wireless security

There are several types of wireless security modes, and you can choose different security modes according to your needs.

• Deactivation (no password)

• WPA-PSK

• WPA2-PSK

• WPAPSKWPA2PSK (i.e. Mixed WPA-PSK and WPA2-PSK)

The interface description is shown in the following table:

| Interface item | Description |
| --- | --- |
| Safe mode | Select WPA-PSK. |
| Encryption type | There are two options: TKIP and AES. |
| WPA-PSK key | Set the key. The legal key length is: 8-63 ASCII characters or 64 hexadecimal numbers (0 ~ 9, a ~ f or A ~ F). |
| Key update interval | Sets the key update interval in seconds. |

- Advanced settings

It is recommended to configure the functional parameters of wireless wifi by default.

- List of wireless clients

A client device accessed via wireless wifi.

- Wireless client MAC filtering

You can set the MAC addresses of wireless devices that are only allowed to access or the MAC addresses of wireless devices that are not allowed to access

The MAC selection item can be used to quickly pull down and select the MAC of the connected device to be added to the list.

- VPN settings

Support two types of VPN tunnel networks: PPTP and L2TP. Enable the required VPN type in the page. Only one of PPTP and L2TP can be enabled. The PPTP and L2TP setup page parameters are the same, using PPTP as an example (L2TP clients do not support IPSec keys)

- Auto Connect: Connect automatically after startup or VPN disconnection

- Only use VPN to connect to the external network: the terminal network data only goes through the VPN tunnel

- VPN server address: supports IP and domain name

- VPN account

- Password for the VPN account

- VPN security mode: the default is auto-negotiation. When auto-negotiation is disabled, you can specify the security mode used by the VPN service

- Automatic connection setting: default 5 times, automatic reconnection after VPN disconnection is detected, 0 value is unlimited times

- Opposite network segment and mask: enable when accessing VPN server intranet application or mutual access application, and fill in the opposite network address segment (non-IP address)

- Disconnection detection: detect whether the VPN gateway link is smooth, and reconnect in case of abnormality

- Network security

- 1. Firewall

- Check Prevent Wan port ping to filter and discard ICMP packets from the Wan port without responding.

- If the firewall is turned off, the relevant filtering and restriction functions will be disabled.

- Modify the TTL value to change the maximum number of nodes that a packet can transmit in the network.

- 2. Site control

By restricting access to domain name websites through keyword filtering, the connected terminal will not be able to access domain name websites containing keywords in the list. If the keyword QQ, will not be able to access the www.qq.com site.

- 3. Access control

Add the IP range of the intranet terminal, the IP address of the destination service, the port number of the destination service, and the time period to the list, and select Pass or Block to control the permission of the intranet terminal to access the destination service.

Pass: only the items configured in the list are allowed to access the destination service; block: access to the destination service is restricted.

As shown in the figure, terminals in the range of 192.168.10.10 to 192.168.10.15 are restricted to access all TCP port services, which means that they cannot access any port application services.

- 4. Port blocking

Add the IP range and port range of the intranet terminal to the list to control the permission of the intranet terminal to access the port service.

As shown in the figure, the terminals in the range of 192.168.10.10 to 192.168.10.12 are restricted to access all the 80-443 port services, which means that they cannot access the 80-443 port WEB services

- System Services

- 1. Virtual service (port mapping)

The external access port can be mapped to the port service of the specified intranet terminal. The preset common application port can be used for quick setting, such as port 80 for WEB application and port 21 for FTP service.

- Preset setting: preset common service ports, which are automatically filled into the following items when selected

- Service name: Customize the service name of this forwarding configuration

- External port: the port visited by the WAN, and the port range is supported

- Internal port: application service port in the intranet, supporting port range

- Network protocol: The protocol used for network data, TCP, UDP, TCP & UDP

- Intranet host: the host IP that provides application services in the intranet

- 2、DMZ

The specified end host in the intranet can be completely exposed to the WAN, and the services accessed by the WAN can be forwarded to the specified end host when they are not directed.

- Enable the DMZ

- Specifies the IP address to forward to the host

- 3. Serial port service

The lower computer device is connected to the serial port of the device through serial port protocol (RS232/RS485), and the data information transmitted by the lower computer device can be forwarded to the specified TCP & UDP server after the serial port service of the device is configured.

The serial port service of the equipment supports the data transparent transmission mode and the command mode. The data transparent transmission mode: the data is transmitted between the serial port and the server without changing the structure and type of the data. The command mode: the at command set specified by the equipment is used to query, set application parameters and send data.

The TCP & UDP of the serial port service of the device supports two modes: client mode and server mode. The client mode forwards data to the specified TCP & UDP server. The server mode accepts connections from other clients and forwards data to the serial port of the device.

Service parameters:

- Host ID: The content sent for the first time after TCP & UDP connection is successful. It is sent by reading the MAC address of the device by default. The content can be filled in by customization.

- Restart time: cycle countdown to restart the serial port service

- Content of heartbeat data: the data sent by cyclic heartbeat. It is sent by reading the MAC address of the device and the IP address of the Wan port by default. The content can be customized.

- Heartbeat time: the time interval for circularly sending the heartbeat data content

- TCP/UDP No Data: Check whether the server has a timeout period for data return, and restart the serial port service if the timeout period is exceeded

- Restart of TCP/UDP without data: Restart the device after continuous detection of timeout and restart of serial port service for the set number of times

Server mode: The server IP address is the gateway address (such as 192.168.10.1) or the Wan port IP

Client mode: support simultaneous connection of up to 5 TCP & UDP servers, support IP address and domain name address

Serial port parameters:

- Baud rate: it shall be consistent with the baud rate used by the lower computer

- Cache policy:

  - No policy: data is sent and received

  - Delay waiting: The serial port receives and waits for the set delay time before forwarding the data.

  - Byte count: the serial port forwards the data after receiving the set number of bytes

  - Delay waiting + byte count: data is forwarded when any one of the set delay time and the set byte count is reached first

  - Specific characters: the serial port receives the data containing specific characters and then forwards the data

- 4. Remote maintenance platform

The device supports data monitoring and remote operation of the cloud platform. The device reports the running status data to the cloud platform through heartbeat, and supports remote upgrade, remote restart, log upload, etc.

- 5. DO control

The equipment supports two independent single-wire DO controllers, which can be used to control the power on and off of peripheral low-power equipment, or the on and off of other types of lines. It supports setting the default on-off state of the DO controller, timing, and switching the on-off state. Open and close: open, close: open.

1) Cycle timing action: change the opening and closing state of DO once after each timing. If the current DO1 is in the opening and closing state, switch DO1 to the closing state after one timing is over, and switch to the opening and opening state after the next timing is over.

2) Timed action: switch DO on/off status at the timed time point. If the current DO2 is on/off status, switch DO2 to on status at 8: 00am on Tuesday, and switch DO2 to on/off state again at 8: 00am on Friday. The device will not be consistent with the current time until the Internet time is synchronized successfully.

3) Synchronous reset: switch the state of another DO to the default on-off state at the same time, and only support one-way synchronous reset.

- Equipment management

- 1. Equipment management

- UPNP: Enable the function to facilitate the transparent transmission of application services

- Remote WEB: Enable and specify the external access port to remotely log in to the WEB management page through the WAN

- Telnet/SSHD: Enable this option to log in to the background environment of the system by using the command line window or terminal tool

- SNMP management: network management protocol

- System log: enable this item to record detailed operation log, which is generally used for fault diagnosis and is disabled by default

- Detect 3G/4G hardware devices: try to restart the system to repair when the 4G hardware module is abnormal and cannot be identified

- Timing restart: set the time interval of automatic cycle restart of the system in minutes.

- Timed restart: Set the system to restart regularly at the specified cycle time, and synchronize the Internet time.

- 2. Time zone management

- Time Zone: The default time is Beijing Time East Zone 8. For other areas, you need to select the corresponding time zone to match the time.

- NTP server: The default is to use the built-in NTP server to synchronize the time. You can select to fill in the NTP server by yourself

- Retry Times: Stop synchronization when the set synchronization times fail

- 3. Backup/Recovery

- Backup: Save the current configuration parameter values as files and download them to the local

- Restore: Restore to the parameter values in the configuration films

- Select File: Configuration file to be restored

- 4. Software upgrade

- Select an upgrade package file with the same model as the device. Upgrading a file with a model other than the device will cause the device to fail to start

- Whether to restore to the factory default value after upgrading, two conditions must be met at the same time:

- The version number of the software to be upgraded is different from the current version number

- Click the Restore Factory Settings tab to cancel the status: Restore Factory Settings after Upgrading Cancel

Note: Do not disconnect the connected network cable during the upgrade process, otherwise it will cause abnormal failure to start normally. It is recommended to upgrade within 3 minutes after the system is powered on and runs normally.

- 5. Restore the factory value

Restore Factory Defaults clears the current setup information for this device and restores it to its original state.

There are two modes of operation:

- Click the Restore Factory Values button on the web management-device management page

- Long press the rest button on the shell for more than 10 seconds, and the syslight will flash to release it, waiting for the system to restart.

- 6. Password management

The default login user name of the equipment cannot be modified, and the password can be modified. For security reasons, it is recommended to change the default password and keep the password information.

Default username: admin Default password: admin

- FAQ

- 1. After the PC is connected, the network card of the computer is notGet the IP

- Check whether the network cable connection is in good contact and whether the LAN lamp flashes.

- Check whether the network card of the computer is set with a static IP and is not in the same network segment as the device.

- 2. Unable to access the Internet after connecting to the PC

- In the default 3G/4G mode, the SIM tariff card needs to be installed for use. After ensuring that the SIM card with normal tariff and status has been installed, enter the web management-current status page to check the 4G status information and confirm whether the signal strength, SIM card status, 4G service and 4G network type are displayed normally. If one of the items is abnormal, check whether the SIM card is inserted or whether the charge of the SIM card is normal.

- Check whether the computer network card is set with a static IP and is not in the same network segment as the device, and whether it is not set or an invalid DNS address is set.

- Whether the computer has installed protection software and enabled similar network interception and filtering functions.

- 3. After the PC is connected, the speed of accessing the Internet is slow and slow.

- The network speed is greatly affected by the local wireless signal. You can enter the web management-current status page to check the 4G signal strength, and move the device to a location with high signal strength when the signal is weak (less than 35%).

- Whether the SIM tariff card has a speed limit, please consult the SIM tariff card operator.

- Whether the SIM tariff card APN access point is not universal. Consult the SIM tariff card operator and compare it with the APN, user name and password information used in the web management 4G setting page to see whether they are the same. If they are not the same, modify them to the information provided by SIM tariff card operator.

- 4. After the PC is connected, the network is intermittently disconnected and then restored.

- Enter the web management-current status page to check whether the Internet connection has been successfully dialed.

- Check whether the IP address for disconnection detection is modified and set in the 4G Settings-Disconnection Detection page. Run the command line window on the computer side to ping the IP address in the host address to check whether the returned result is successful. If it fails or times out, it indicates that the network is abnormal, and the system automatically tries to restore the network. The disconnection detection function will not be affected after it is turned off.

- After the disconnection detection is enabled, it is necessary to ensure that the IP in the detected host address can respond normally and the returned result is normal.